# Technical Information

# Care and Maintenance of finished floors

Finished wooden floors such as parquet, wood block paving or planks are really representative floors. Their main advantages are the high wear resistance and durability. Our original swedish finish has been used all over the world for many years on many wooden floors.

These high quality products give a shiny surface to your wooden floors, keep the wood attractive and protect the floor against dirt and wear. Finished parquet floors are hygienic and easy to care.

Please note:

Wood is a natural material which is influenced by both, temperature and humidity. In case of high humidity the floor absorbs moisture and expands. In relatively dry environments (such as during the winter when heating systems are being used), moisture evaporates from the floor and it shrinks. Gaps occur.

This is a natural and continuous process. Appropriate room temperature and humidity especially during the heating season is recommended.

This is beneficial not only to your parquet floor but also to the well being and to the health of the inhabitants.

The ideal room climate is 18-20°C/50-60% RH.

After application of the finish the floor should not be walked on for a few days, if possible. The remaining humidity in the room will disappear by regular heating and fresh air. This ensures that the floor will not be affected negatively. In newly built houses wood deformations are possible due to humidity, but will disappear within a short time. The application of the finish on wooden floors is done manually, therefore slight spots or unevenness cannot be avoided totally. These surface appearances will disappear during the further floor use.

Please consider: The surface treatment of parquet and other wooden floors with any finish will not avoid the appearance of gaps. The finish protects the wood against wear and makes it possible to maintain the floor easy and cost effective.

The finish protects the floor with a closed film coat against penetrating water and dirt.

Every finishing system will be affected by wear to some degree, depending upon traffic levels. Dust and dirt wear off the finish. Regular care and maintenance reduce the signs of wear.

Please read the following instructions carefully:

14 days careful treatment

Let the finish dry for about 8-12 hours, then the floor can be walked on again. During the next 14 days treat the floor carefully. Do not use any water for cleaning.

# Start of the regular care

Start the regular care after 14 days.

Cleaning:

Regular cleaning keeps the floor hygienic. It is carried out either dry with a mop, soft broom, vacuum cleaner or it is wiped with a well squeezed damp mop using Bona Parquet Cleaner.

Maintenance:

Maintenance depends on wear of the floor. It can be a substitute for cleaning. Wipe the floor with a polish/water solution, approximately 400-500 ml Bona Parquet Polish to 10 litres of water.  This procedure stops the gradual build-up of too many polish coats and removes dust, sand and any loose dirt.

Regular periodic maintenance:

Easy to do!

The first regular maintenance treatment should be carried out before daily use of the floor, but at least 10-14 days after the last finishing coat. Apply Bona Parquet Polish with a polish applicator even and thin on the floor. Rubber marks or fat stains can be wiped out with Bona Parquet Cleaner.

Depending on traffic a regular maintenance should be carried out every 2-4 weeks. A thin polish film should always be on the floor.

Basic treatment:

A basic treatment should be carried out manually or with a buffing machine if the floor shows dirt or marks that cannot be removed through normal cleaning methods or if polish coats have built up. We recommend Bona Parquet Polish Remover for this procedure. It is important to keep the amount of water introduced to the wooden floor to a minimum to avoid water damages through wood expansion. Work in sections and remove excess cleaning solution immediately. After that apply Bona Parquet Polish on the floor.

Important advice:

Effective and properly dimensioned walk-off mats in the entrance area will make the maintenance of the sealed floor easier and prolong the life of the finish.

Bona GmbH, Jahnstr. 12

65549 Limburg

Tel: 0 64 31 - 40 08-0 Fax: -25

BonaKemi AB, NL Deutschland

Jahnstr. 12

65549 Limburg

Tel: 0 64 31 - 40 08-0 Fax: -25